High-Current 3-State Outputs Drive Bus

Lines Directly or up to 15 LSTTL Loads

Bus-Structured Pinout

SN54HC573A, SN74HC573A OCTAL TRANSPARENT D-TYPE LATCHES WITH 3-STATE OUTPUTS

SCLS147C – DECEMBER 1982 – REVISED MARCH 2001

SN54HC573A ...J OR W PACKAGE SN74HC573A . . . DB, DW, N, OR PW PACKAGE

(TOP VIEW)

description

These octal transparent D-type latches feature

3-state outputs designed specifically for driving highly  capacitive  or  relatively  low-impedance loads.    They    are    particularly    suitable    for implementing    buffer    registers,    I/O    ports, bidirectional bus drivers, and working registers. While the latch-enable (LE) input is high, the Q outputs respond to the data (D) inputs. When LE is low, the outputs are latched to retain the data

OE

1D

2D

3D

4D

5D

6D

7D

8D GND

1

2

3

4

5

6

7

8

9

10

20

19

18

17

16

15

14

13

12

11

VCC

1Q

2Q

3Q

4Q

5Q

6Q

7Q

8Q LE

that was set up.

A buffered output-enable (OE) input can be used to place the eight outputs in either a normal logic state   (high   or   low   logic   levels)   or   the high-impedance  state.  In  the  high-impedance

SN54HC573A . . . FK PACKAGE

(TOP VIEW)

3   2   1   20 19

4

state, the outputs neither load nor drive the bus lines significantly. The high-impedance state and increased drive provide the capability to drive bus lines without interface or pullup components.

OE does not affect the internal operations of the latches. Old data can be retained or new data can be   entered   while   the   outputs   are   in   the high-impedance state.

3D

4D

5D

6D

7D

5

6

7

8

18

17

16

15

14

9 10  11 12 13

2Q

3Q

4Q

5Q

6Q

ORDERING INFORMATION

TA	PACKAGE†		ORDERABLE PART NUMBER

TOP-SIDE MARKING

PDIP – N	Tube	SN74HC573AN	SN74HC573AN

Tube	SN74HC573ADW

–40C to 85C

SOIC – DW

Tape and reel	SN74HC573ADWR

HC573A

SSOP – DB	Tape and reel	SN74HC573ADBR	HC573A TSSOP – PW	Tape and reel	SN74HC573APWR	HC573A

CDIP – J	Tube	SNJ54HC573AJ	SNJ54HC573AJ

–55C to 125C	CFP – W	Tube	SNJ54HC573AW	SNJ54HC573AW

LCCC - FK	Tube	SNJ54HC573AFK	SNJ54HC573AFK

† Package drawings, standard packing quantities, thermal data, symbolization, and PCB  design guidelines are available at www.ti.com/sc/package.

Please be aware that an important notice concerning availability,  standard warranty, and use in critical  applications of

Texas Instruments semiconductor products and disclaimers thereto appears at the end of this data sheet.

PRODUCTION  DATA  information  is  current  as  of  publication  date. Products conform to specifications per the terms of Texas Instruments standard warranty. Production processing does not necessarily include testing of all parameters.

Copyright  2001, Texas Instruments Incorporated

On products compliant to MIL-PRF-38535, all parameters are tested unless  otherwise  noted.  On  all  other  products,  production processing does not necessarily include testing of all parameters.

POST OFFICE BOX 655303  DALLAS, TEXAS 75265

1

SN54HC573A, SN74HC573A

OCTAL TRANSPARENT D-TYPE LATCHES WITH 3-STATE OUTPUTS

SCLS147C – DECEMBER 1982 – REVISED MARCH 2001

FUNCTION TABLE

(each latch)

INPUTS	OUTPUT

OE	LE	D	Q

L	H	H	H

L	H           L	 L	 L	 L           X	Q0

H	X	X	Z

logic symbol†

1

OE

11

LE

2

1D

3

2D

4

3D

5

4D

6

5D

7

6D

8

7D

9

8D

EN

C1

1D

19

18

17

16

15

14

13

12

1Q

2Q

3Q

4Q

5Q

6Q

7Q

8Q

† This symbol is in accordance with ANSI/IEEE Std 91-1984 and IEC Publication 617-12.

logic diagram (positive logic)

OE

1

LE

11

1D

2

C1

1D

19

1Q

To Seven Other Channels

2	POST OFFICE BOX 655303  DALLAS, TEXAS 75265

SN54HC573A, SN74HC573A OCTAL TRANSPARENT D-TYPE LATCHES WITH 3-STATE OUTPUTS

SCLS147C – DECEMBER 1982 – REVISED MARCH 2001

absolute maximum ratings over operating free-air temperature range†

Supply voltage range, VCC

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

–0.5 V to 7 V

Input clamp current, IIK (VI < 0 or VI > VCC) (see Note 1)

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

20 mA

Output clamp current, IOK (VO < 0 or VO > VCC) (see Note 1)

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

20 mA

Continuous output current, IO (VO = 0 to VCC)

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

35 mA

Continuous current through VCC or GND

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

70 mA

Package thermal impedance, JA (see Note 2): DB package

DW package

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

70C/W

58C/W

N package

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

69C/W

PW package

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

83C/W

Storage temperature range, Tstg

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

–65C to 150C

† Stresses beyond those listed under “absolute maximum ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated under “recommended operating conditions” is not implied. Exposure to absolute-maximum-rated conditions for extended periods may affect device reliability.

NOTES:   1.   The input and output voltage ratings may be exceeded if the input and output current ratings are observed.

2.   The package thermal impedance is calculated in accordance with JESD 51-7.

recommended operating conditions

SN54HC573A	SN74HC573A

MIN	NOM	MAX	MIN	NOM	MAX

UNIT

VCC	Supply voltage	2	5	6	2	5	6	V

VCC = 2 V	1.5	1.5

VIH	High-level input voltage	VCC = 4.5 V	3.15	3.15	V VCC = 6 V	  4.2	  4.2

VCC = 2 V	0	0.5	0	0.5

VIL	Low-level input voltage	VCC = 4.5 V	0	1.35	0	1.35	V VCC = 6 V	0		1.8	0		1.8

VI	Input voltage	0	VCC	0	VCC	V VO	Output voltage	0	VCC	0	VCC	V

VCC = 2 V	0	1000	0	1000

tt	Input transition (rise and fall) time	VCC = 4.5 V	0	500	0	500	ns

VCC = 6 V	0	400	0	400

TA	Operating free-air temperature	–55	125	–40	85	C

POST OFFICE BOX 655303  DALLAS, TEXAS 75265

3

SN54HC573A, SN74HC573A

OCTAL TRANSPARENT D-TYPE LATCHES WITH 3-STATE OUTPUTS

SCLS147C – DECEMBER 1982 – REVISED MARCH 2001

electrical  characteristics  over  recommended  operating  free-air  temperature  range  (unless otherwise noted)

PARAMETER	TEST CONDITIONS	VCC

TA = 25C	SN54HC573A	SN74HC573A

MIN	TYP	MAX	MIN	MAX	MIN	MAX

UNIT

2 V	1.9	1.998	1.9	1.9

IOH = –20 A	4.5 V	4.4	4.499	4.4	4.4

VOH	VI = VIH or VIL			6 V		5.9	5.999	5.9	  5.9	V IOH = –6 mA	4.5 V	3.98		4.3	3.7	3.84

IOH = –7.8 mA	6 V	5.48	5.8	5.2	5.34

2 V	0.002	0.1	0.1	0.1

IOL = 20 A	4.5 V	0.001	0.1	0.1	0.1

VOL	VI = VIH or VIL			6 V	0.001	  0.1	0.1	  0.1	V IOL = 6 mA	4.5 V	  0.17	0.26	0.4	0.33

IOL = 7.8 mA	6 V	0.15	0.26	0.4	0.33

II	VI = VCC or 0			6 V	  0.1	100	1000	1000	nA IOZ	VO = VCC or 0			6 V	0.01		0.5		10		5	A ICC	VI = VCC or 0,	IO = 0		6 V					8		160		80	A Ci			2 V to 6 V		3		   10		  10		10	 pF

timing requirements over recommended operating free-air temperature range (unless otherwise noted)

VCC

TA = 25C	SN54HC573A	SN74HC573A

MIN	MAX	MIN	MAX	MIN	MAX

UNIT

2 V	80	120	100

tw	Pulse duration, LE high	4.5 V	16	24	20	ns

6 V	14	20	17

2 V	50	75	63

tsu	Setup time, data before LE	4.5 V	10	15	13	ns

6 V	9	13	11

2 V	20	24	24

th	Hold time, data after LE	4.5 V	5	5	5	ns

6 V	5	5	5

4	POST OFFICE BOX 655303  DALLAS, TEXAS 75265

SN54HC573A, SN74HC573A OCTAL TRANSPARENT D-TYPE LATCHES WITH 3-STATE OUTPUTS

SCLS147C – DECEMBER 1982 – REVISED MARCH 2001

switching characteristics over recommended operating free-air temperature range, CL = 50 pF

(unless otherwise noted) (see Figure 1)

PARAMETER	FROM	TO	VCC

TA = 25C	SN54HC573A	SN74HC573A

UNIT

t  dpd

(INPUT)	(OUTPUT)	VCC	MIN	TYP	MAX	MIN	MAX	MIN	MAX	UNIT

2 V	77	175	265	220

D	Q	4.5 V	26	35	53	44

6 V	23	30	45	38

ns

2 V	87	175	265	220

LE	Any Q	4.5 V	27	35	53	44

6 V	23	30	45	38

2 V	68	150	225	190

ten	OE	Any Q	4.5 V	24	30	45	38	ns

6 V	21	26	38	32

2 V	47	150	225	190

tdis	OE	Any Q	4.5 V	23	30	45	38	ns

6 V	21	26	38	32

2 V	28	60	90	75

tt	Any Q	4.5 V	8	12	18	15	ns

6 V	6	10	15	13

switching characteristics over recommended operating free-air temperature range, CL = 150 pF

(unless otherwise noted) (see Figure 1)

PARAMETER	FROM	TO	VCC

TA = 25C	SN54HC573A	SN74HC573A

UNIT

t  dpd

(INPUT)	(OUTPUT)	VCC	MIN	TYP	MAX	MIN	MAX	MIN	MAX	UNIT

2 V	95	200	300	250

D	Q	4.5 V	33	40	60	50

6 V	21	34	51	43

ns

2 V	103	225	335	285

LE	Any Q	4.5 V	33	45	67	57

6 V	29	38	57	48

2 V	85	200	300	250

ten	OE	Any Q	4.5 V	29	40	60	50	ns

6 V	26	34	51	43

2 V	60	210	315	265

tt	Any Q	4.5 V	17	42	63	53	ns

6 V	14	36	53	45

operating characteristics, TA = 25C

PARAMETER	TEST CONDITIONS	TYP	UNIT

Cpd	Power dissipation capacitance per latch	No load	50	pF

POST OFFICE BOX 655303  DALLAS, TEXAS 75265

5

SN54HC573A, SN74HC573A

OCTAL TRANSPARENT D-TYPE LATCHES WITH 3-STATE OUTPUTS

SCLS147C – DECEMBER 1982 – REVISED MARCH 2001

PARAMETER MEASUREMENT INFORMATION

S1

VCC

PARAMETER

RL

CL

50 pF

S1

S2

Test

t

tPZH  	Open	Closed

From Output

Under Test

CL

Point

RL

en

tdis

tPZL

tPHZ

1 k

1 k

or

150 pF

50 pF

Closed	Open

Open	Closed

(see Note A)

LOAD CIRCUIT

S2

tPLZ

tpd or tt

––

50 pF

or

150 pF

Closed	Open

Open	Open

High-Level

Pulse  		50%	50%

tw

VCC

0 V

VCC

Reference

Input

Data

Input

50%

10%

tsu

90%

50%

th

90%

50%

10%

VCC

0 V

VCC

0 V

Low-Level

Pulse

50%	50%

VOLTAGE WAVEFORMS PULSE DURATIONS

0 V

tr	tf

VOLTAGE WAVEFORMS

SETUP AND HOLD AND INPUT RISE AND FALL TIMES

Input

50%

50%

VCC

0 V

Output

Control

(Low-Level

50%	50%

VCC

tPLH	tPHL

VOH

Enabling)

tPZL	tPLZ

0 V

In-Phase

Output

50%

10%

90%	90%

50%

10% VOL

Output

Waveform 1

VCC

50%

VCC

tr	tf

tPHL	tPLH

(See Note B)

10%

VOL

Out-of-

90%

50%	50%

90%

VOH

tPZH	tPHZ Output

VOH

Phase

Output

10%	10%

tf

VOLTAGE WAVEFORMS

tr

VOL

Waveform 2

(See Note B)

50%

VOLTAGE WAVEFORMS

90%

0 V

PROPAGATION DELAY AND OUTPUT TRANSITION TIMES

ENABLE AND DISABLE TIMES FOR 3-STATE OUTPUTS

NOTES:   A.   CL includes probe and test-fixture capacitance.

B.   Waveform 1 is for an output with internal conditions such that the output is low except when disabled by the output control. Waveform 2 is for an output with internal conditions such that the output is high except when disabled by the output control.

C.   Phase relationships between waveforms were chosen arbitrarily. All input pulses are supplied by generators having the following characteristics: PRR  1 MHz, ZO = 50 , tr = 6 ns, tf = 6 ns.

D.   The outputs are measured one at a time with one input transition per measurement. E.   tPLZ and tPHZ are the same as tdis.

F.   tPZL and tPZH are the same as ten. G.   tPLH and tPHL are the same as tpd.

Figure 1. Load Circuit and Voltage Waveforms

6	POST OFFICE BOX 655303  DALLAS, TEXAS 75265